# FireSafe Vision — Property Assessment Framework

A structured decision framework for AI-assisted wildfire resilience assessment Sources: CAL FIRE Public Resource Code 4291, California FireSafe Council, UC Cooperative Extension, California Native Plant Society (CalScape)

## Purpose

This document defines the structured logic FireSafe Vision uses to translate a property photo and intake form into a zone-specific assessment and recommendation set. It is the bridge between the zone rules (calfire-zone-rules.md), the plant database (socal-plant-list.md), and the AI prompt layer.

## Assessment inputs

### From photo analysis (AI vision)

The vision model reads the uploaded property photo and identifies:

- Approximate zone being photographed (0–5 ft, 5–30 ft, 30–100 ft)

- Visible surface material (gravel, mulch, concrete, bare soil, grass, pavers)

- Vegetation presence and approximate density in Zone 0

- Fence material (wood, metal, vinyl, masonry)

- Slope presence and estimated gradient

- Proximity of vegetation to the structure

- Visible debris accumulation (gutters, eaves, corners)

- Presence of combustible items (wood piles, furniture, propane)

### From intake form

The form collects what the photo cannot reliably determine:

| Field | Options |
| --- | --- |
| Zone to assess | Zone 0 (0–5 ft), Zone 1 (5–30 ft), Zone 2 (30–100 ft), Full property |
| Topography | Flat, Mild slope (<20%), Moderate slope (20–40%), Steep slope (>40%), Canyon, Ridgetop |
| Property type | Single-family home, ADU/detached structure, Rural/WUI parcel |
| Current surface material (Zone 0) | Mulch/bark, Lawn/grass, Dirt/bare soil, Gravel, Concrete/pavers, Mixed |
| Current fence material | Wood, Vinyl, Metal, Masonry/concrete, None |
| Irrigation present | Yes — automated, Yes — manual, No |
| Fire weather zone | High, Very High, Extreme (based on CAL FIRE Hazard Severity Zone map) |

## Hazard scoring logic

Each property condition is assigned a hazard flag. Flags are used to prioritize recommendations in the output report.

### Zone 0 hazard flags

| Condition | Hazard level | Flag |
| --- | --- | --- |
| Combustible mulch or bark within 5 ft of structure | High | Z0-MULCH |
| Wood fence attached to structure | High | Z0-FENCE |
| Vegetation within 0–5 ft of structure | High | Z0-VEG |
| Debris visible in gutters or eaves | High | Z0-GUTTER |
| Combustible furniture or items visible | Medium | Z0-ITEMS |
| Lawn/grass within 5 ft | Medium | Z0-LAWN |
| No non-combustible perimeter visible | High | Z0-SURFACE |

### Zone 1 hazard flags

| Condition | Hazard level | Flag |
| --- | --- | --- |
| Dense shrubs touching or near structure | High | Z1-DENSE |
| Visible ladder fuels (shrubs under trees) | High | Z1-LADDER |
| Unseparated plantings — no spacing visible | Medium | Z1-SPACING |
| Trees with low limbs (below 6 ft) | Medium | Z1-LIMB |
| Dead or dry vegetation present | High | Z1-DEAD |
| Slope present without increased spacing | Medium | Z1-SLOPE |

### Zone 2 hazard flags

| Condition | Hazard level | Flag |
| --- | --- | --- |
| Tree canopies touching or overlapping | Medium | Z2-CANOPY |
| Dead or dying trees visible | High | Z2-DEAD |
| Slope > 40% with grouped plantings | High | Z2-STEEP |
| No visible limbing on trees | Low | Z2-LIMB |

## Recommendation generation logic

Recommendations are generated by matching active hazard flags to the action library below. Each recommendation includes: the action, the rationale, the material or plant option, and a difficulty/cost tier.

### Action library — Zone 0

Z0-SURFACE — Non-combustible ground surface

- Action: Replace combustible surface material with non-combustible hardscaping within 5 ft of all structures

- Options: 3/4-inch crushed gravel, decomposed granite, concrete, brick, flagstone, pervious pavers

- Rationale: Zone 0 surface material is the primary path for ember ignition — eliminating combustible surface material is the single highest-impact action

- Cost tier: Low–Medium | DIY-accessible

Z0-MULCH — Remove combustible mulch

- Action: Remove bark mulch and wood chip mulch from Zone 0 entirely

- Replacement: Inorganic gravel or decomposed granite

- Rationale: Bark mulch acts as an ember bed — it catches and holds embers until ignition

- Cost tier: Low | DIY-accessible

Z0-VEG — Remove Zone 0 vegetation

- Action: Remove all plants, shrubs, and organic ground cover from within 5 ft of the structure

- Exception: Low-growing, high-moisture succulents (e.g., Dudleya) if consistently irrigated

- Cost tier: Low | DIY-accessible

Z0-FENCE — Replace combustible fencing

- Action: Replace wood or vinyl fence within Zone 0 with non-combustible material

- Options: Aluminum, steel, wrought iron, concrete block, masonry, fiber cement panels

- At minimum: Install a non-combustible "gate section" of at least 5 ft adjacent to the structure

- Cost tier: Medium–High | Contractor recommended

Z0-GUTTER — Clear gutters and eaves

- Action: Remove all leaf litter, pine needles, and debris from gutters, eaves, and roof valleys

- Upgrade: Install metal mesh gutter guards (ember-resistant) — look for California-approved WUI vent products

- Frequency: Clear before fire season; after every significant wind event

- Cost tier: Low | DIY-accessible

Z0-ITEMS — Remove combustible items

- Action: Remove or relocate all combustible items stored under decks, on porches, or within Zone 0

- Items to remove: Firewood, propane tanks, furniture, doormats, potted plants with combustible pots

- Cost tier: None | Immediate action

### Action library — Zone 1

Z1-DEAD — Remove dead and dry vegetation

- Action: Immediately remove all dead plants, dried grasses, dead branches, and accumulated debris

- Priority: Highest — dead material ignites at much lower temperatures and spreads fire rapidly

- Cost tier: Low | DIY-accessible with tools

Z1-LADDER — Eliminate ladder fuels

- Action: Remove shrubs and small trees growing beneath larger trees — these create a "ladder" that carries fire from ground to canopy

- Rule: Maintain vertical clearance of 3× the height of the tallest shrub between shrub tops and lowest tree branches

- Cost tier: Low–Medium | Chainsaw work may require contractor

Z1-SPACING — Increase plant spacing

- Action: Thin and space existing plants to prevent fire from spreading plant to plant

- Flat ground: Minimum 2× shrub height between shrubs; 10 ft between tree canopy edges

- Sloped ground: Minimum 3× shrub height between shrubs; 30 ft between tree canopy edges

- Plant recommendations: Replace removed plants with species from the Zone 1 plant list

- Cost tier: Low–Medium | DIY-accessible

Z1-LIMB — Limb up trees

- Action: Remove lower branches from all trees to a minimum height of 6 ft from the ground

- On slopes: Increase to 8–10 ft minimum clearance

- Note: Prioritize tree health — do not remove more than one-third of live branches in a single season

- Cost tier: Low–Medium | Arborist recommended for large trees

Z1-SLOPE — Address slope-specific spacing

- Action: Increase spacing distances based on slope percentage — see slope modifier table in zone rules

- Additional action: Consider terraced garden walls to create maintainable planting pockets and firefighter access

- Plant selection: Choose deep-rooted species for slope stabilization — see slope stabilization plant list

- Cost tier: Medium–High | Landscape contractor recommended

### Action library — Zone 2

Z2-DEAD — Remove dead or dying trees

- Action: Identify and promptly remove any dead, dying, or structurally compromised trees

- Warning signs: Excessive leaf loss, bark loss, fungal growth, excessive dryness

- Note: Do not wait — a falling dead tree or one that ignites easily is a significant risk

- Cost tier: Medium–High | Licensed arborist required for large trees

Z2-CANOPY — Separate overlapping canopies

- Action: Thin canopies to achieve minimum separation per zone distance rules

- 5–30 ft: 18 ft minimum between canopy edges

- 30–60 ft: 12 ft minimum

- 60–100 ft: 6 ft minimum

- Method: Selective limbing and thinning — do not top trees

- Cost tier: Medium | Arborist recommended

Z2-STEEP — Steep slope management

- Action: On slopes greater than 40%, eliminate grouped plantings — single plants only

- Where vegetation cannot be maintained: Install non-combustible retaining wall or gabion structure

- Cost tier: Medium–High | Contractor required for structural walls

## Report output structure

Each generated report includes the following sections in order:

- Property photo — the original uploaded image

- FireSafe Vision image — the AI-generated transformed view

- Hazard summary — flagged conditions with severity (High / Medium / Low)

- Zone-by-zone action plan — prioritized by hazard level, with specific material and plant options

- Plant recommendations — 3–5 specific species appropriate for the property's zone, topography, and conditions

- Maintenance reminders — seasonal tasks relevant to the flagged conditions

- Next steps — pointer to CAL FIRE inspections, local FireSafe Council, and professional directory

## Prompt construction logic

The AI prompt assembles as follows:

System context:

You are a wildfire resilience assessment assistant.

Analyze the attached property photo and the intake form data.

Apply the hazard scoring framework to identify active hazard flags.

Return a structured JSON object with: zone, active_flags[], hazard_level, recommendations[].

Base all recommendations on CAL FIRE Public Resource Code 4291 defensible space standards.

Retrieved context (RAG):

[Relevant chunks from calfire-zone-rules.md and socal-plant-list.md

matched to the detected zone and topography type]

User input:

Photo: [base64 encoded image]

Intake form: {zone, topography, surface_material, fence_material, irrigation, fire_weather_zone}

Expected output schema:

{

"zone_assessed": "Zone 0",

"topography": "mild_slope",

"active_flags": [

{ "flag": "Z0-MULCH", "severity": "high", "observed": "bark mulch visible within 3 ft of foundation" },

{ "flag": "Z0-FENCE", "severity": "high", "observed": "wood fence attached to structure" }

],

"overall_risk": "high",

"priority_actions": [

{

"flag": "Z0-MULCH",

"action": "Remove bark mulch from Zone 0",

"replacement": "3/4-inch crushed gravel or decomposed granite",

"rationale": "Mulch acts as an ember bed — highest-impact single action in Zone 0",

"cost_tier": "low",

"diy_accessible": true

}

],

"plant_recommendations": [

{

"name": "Chalk dudleya",

"scientific": "Dudleya pulverulenta",

"zone": "Zone 1",

"notes": "Low-growing native succulent; excellent fire resistance; minimal maintenance"

}

]

}

## Jurisdiction scope — v1

FireSafe Vision v1 is scoped to:

- Los Angeles County — unincorporated areas and municipalities

- Primary focus communities: Altadena, Pasadena, Arcadia, Sierra Madre, Monrovia, Duarte

- Fire Hazard Severity Zones: High, Very High, and Extreme as mapped by CAL FIRE

Recommendations reference California state law (PRC 4291). Local municipal codes may impose additional requirements — the report includes a reminder to verify local requirements with the applicable fire authority.

To adapt for another jurisdiction, update:

- knowledge-base/calfire-zone-rules.md — add or replace with local fire code

- knowledge-base/socal-plant-list.md — replace with regionally appropriate plant species

- lib/rag.ts — update jurisdiction tag in the retrieval filter

## Framework versioning

| Version | Date | Changes |
| --- | --- | --- |
| 1.0 | May 2026 | Initial framework — LA County scope, Zones 0–2, SoCal plant list |

Framework updates should be reviewed against the current CAL FIRE defensible space guidelines annually and after any significant regulatory changes (e.g., Zone Zero executive order implementation).