# Defensible Space Zone Rules

Source: CAL FIRE — California Department of Forestry and Fire Protection Reference: Public Resource Code 4291 | fire.ca.gov/defensible-space

## Overview

Defensible space is the buffer a homeowner creates between a building and the vegetation, trees, and other plants surrounding it. A well-maintained defensible space gives a home a better chance of surviving a wildfire and gives firefighters a safer environment to defend the structure.

California law requires 100 feet of defensible space for homes located in State Responsibility Areas (SRA). Local jurisdictions may require additional clearance.

Zone 0 specifically is governed by two separate standards that do not move on the same timeline: a state fire-code mandate (still in draft) and a private insurance-industry standard (active today, tied to premium discounts). Both are documented below — do not treat them as one unified rule. See "Zone 0 — Regulatory vs. Insurance Standard" for the full breakdown.

## Zone 0 — The Home Ignition Zone (0–5 feet)

Zone 0 addresses the area immediately surrounding the structure. Research consistently shows that most homes ignite not from direct flame contact but from ember accumulation in this zone.

### Regulatory status (as of August 2026)

- State fire code (PRC § 4291 / AB 3074): The Board of Forestry & Fire Protection has not finalized a statewide Zone 0 regulation. AB 3074 (2020) directed the Board to write one; Executive Order N-18-25 set a December 31, 2025 adoption deadline; that deadline passed without adoption. The Board's latest draft was released April 17, 2026 (workshop April 23, 2026) and remains a draft. There is currently no statewide, enforceable Zone 0 requirement. Existing PRC 4291 Zone 1 and Zone 2 requirements remain fully in effect and enforceable via SB 63 inspection programs.

- Local ordinances: Some jurisdictions have adopted their own Zone 0 rules ahead of the state — notably the City of San Diego. Where a local ordinance exists, it is enforceable today regardless of state rulemaking status.

- Scope as drafted: Applies primarily to Very High Fire Hazard Severity Zone (FHSZ) parcels in State Responsibility Areas; some Local Responsibility Area parcels fall outside the current draft's scope.

### Insurance standard (active today)

- IBHS Wildfire Prepared Home™ — a voluntary, insurer-backed certification with its own 0–5 ft Noncombustible Zone requirement, required at both the Essential (formerly Base) and Enhanced (formerly Plus) tiers. This requirement is stricter than the state's draft rule — zero vegetation tolerance, no exceptions.

- This standard is made financially meaningful by 10 CCR § 2644.9 ("Safer from Wildfires"), effective since October 2022 and expanded by AB 1 (effective January 1, 2026), which legally requires California insurers pricing wildfire risk to offer documented discounts for these mitigation actions. Documented discounts range roughly 5–35% depending on carrier.

- Because this standard is active and financially actionable right now, it should be the primary standard FireSafe Vision measures Zone 0 photos against — with the state draft surfaced as "anticipated" context, not a current requirement.

### Goal

Eliminate all combustible material. Create a non-combustible perimeter. (This goal is shared by both tracks — the difference is in enforceability and in how strictly "no vegetation" is applied; see divergence note below.)

### Surface materials

Meets both tracks

- Gravel, decomposed granite, or crushed rock (3–4 inch depth)

- Concrete, brick, flagstone, or other masonry

- Pervious concrete pavers

- Sand or bare soil as a minimum baseline

### Vegetation

Divergence point — flag explicitly in tool output

- IBHS standard: No vegetation in Zone 0, full stop — no exceptions.

- State draft standard: No vegetation is the ideal condition, but if any plants are present, they must be low-growing, well-irrigated, and separated from the structure (see socal-plant-list.md Zone 0 exceptions, e.g. Dudleya).

- A homeowner pursuing an insurance discount should follow the IBHS standard, not the more lenient state draft language.

- Remove all dead plants, dried leaves, dry grass, and other dead vegetation immediately regardless of track.

### Structures and objects

Meets both tracks

- Remove all combustible items from under decks and porches — firewood, furniture, doormats, planters

- Clear gutters and roof valleys of all debris and leaf litter

- Enclose open areas beneath elevated decks and porches with non-combustible materials

- Remove wood chip or bark mulch within Zone 0 — mulch can act as an ember bed

### Fencing

Meets both tracks

- Fencing attached to the home within Zone 0 must be non-combustible: metal, concrete, masonry, fiber cement

- A non-combustible "gate" section of at least 5 feet adjacent to the structure is the minimum if full non-combustible fencing is not possible

- State draft text (§1298.04(b)(8)–(9)) and local ordinances (e.g., San Diego) may include specific fence exceptions — check jurisdiction

### Vents

IBHS-only requirement — not part of current PRC 4291 / Zone 0 draft text

- IBHS Essential and Enhanced tiers require ember-resistant vents (mesh tested to ASTM E2886) on foundation, gable, soffit, and ridge vents

- State building code (Chapter 7A) addresses ignition-resistant vents for new construction only — not a retrofit mandate for existing homes under PRC 4291

### Utilities

Meets both tracks

- Identify and address gas meter locations relative to the structure

- Clear propane tanks — maintain a 10-foot non-combustible perimeter

## Zone 1 — Lean, Clean, and Green (5–30 feet)

Zone 1 is the area immediately surrounding Zone 0. The goal is to significantly reduce fuel load while maintaining living, well-irrigated plants.

### Goal

Remove dead and dying vegetation. Maintain well-irrigated, fire-resistant plants with appropriate spacing to prevent fire spread from plant to plant and from ground to canopy.

### Vegetation management

- Remove all dead plants, dried grasses, weeds, and leaf litter regularly

- Prune trees to remove dead branches and limbs

- Remove branches that extend over the roof or within 10 feet of the chimney

### Tree spacing

- Maintain a minimum of 10 feet between tree canopies on flat ground

- On slopes, increase spacing — flames travel faster uphill and require greater horizontal separation

- Remove "ladder fuels" — shrubs and smaller trees beneath larger trees that allow fire to climb from ground to canopy

### Shrub spacing

- Space shrubs so that a fire in one shrub cannot easily spread to the next

- On flat ground: space shrubs a minimum of 2 times the height of the shrub apart

- On sloped ground: increase to 3 times the shrub height

### Vertical spacing (limbing up)

- Remove lower branches of trees to a height of at least 6 feet from the ground

- On slopes, increase limb-up height — the steeper the slope, the higher the clearance needed

- Minimum vertical clearance formula: 3× the height of the underlying shrub vegetation

### Ground cover and mulch

- Use composted wood chips (not bark mulch) at a maximum depth of 3 inches

- Keep mulch away from the base of any plant or tree trunk

- Avoid fine, fibrous mulch materials (e.g., gorilla hair) — they ignite rapidly

## Zone 2 — Reduce Fuel (30–100 feet)

Zone 2 addresses the broader landscape beyond the immediate structure zone. The goal is to slow fire spread by reducing overall fuel density.

### Goal

Thin vegetation to slow fire advance and reduce the intensity of a fire approaching the structure. This zone should be low-maintenance relative to Zones 0 and 1.

### Tree spacing — flat sites

- 5–30 ft from structure: minimum 18 feet between tree canopy edges

- 30–60 ft from structure: minimum 12 feet between canopy edges

- 60–100 ft from structure: minimum 6 feet between canopy edges

### Tree spacing — sloped sites

- Slope less than 20%: minimum 20 feet between tree canopies

- Slope 20–40%: minimum 40 feet between tree canopies

- Slope greater than 40%: single trees only — no grouped plantings

### Shrub spacing — sloped sites

- Slope less than 20%: space shrubs 2× their height

- Slope 20–40%: space shrubs 4× their height

- Slope greater than 40%: space shrubs 6× their height, individual plants only

### General Zone 2 maintenance

- Remove all dead and dying trees, shrubs, and vegetation

- Remove branches within 6 feet of the ground on all trees

- Mow or cut annual grasses to a maximum height of 4 inches during dry season

- Address any visible issues — dead or dying trees, collapsed spacing — promptly

## Topography modifiers

Slope, aspect, and local wind exposure significantly increase fire risk. Apply these modifiers on top of standard zone requirements.

| Site condition | Risk increase | Recommended adjustment |
| --- | --- | --- |
| Slope 0–20% | Low | Standard zone distances |
| Slope 20–40% | Moderate | Increase defensible space distances; harden structure |
| Slope 40%+ | High | Maximum hardening; early evacuation planning |
| Canyon or saddle | Very high | Increase Zone 2 distances; limit plantings on uphill side |
| Ridgetop or hilltop | High | Protect all sides equally; increase spacing |
| South/southwest facing | Moderate-high | Higher evapotranspiration — prioritize irrigation |

## Building siting and separation

### Between neighboring structures

- Optimal: 30 feet or more between structures

- If neighbor is also rebuilding: coordinate to place both homes at least 15 feet from shared property line

- If space is constrained: fire-rate the wall(s) closest to the neighboring structure (1-hour minimum)

### Detached structures on property (garages, ADUs)

- Maintain at least 30 feet of separation between structures on the same property

- If less than 30 feet is available, apply additional hardening to the detached structure

### Wildland-Urban Interface (WUI) — open land adjacency

- Maintain at least 50 feet of separation between any structure and open wildland

- If not possible, fire-rate the wall(s) closest to the open land (1-hour minimum)

## Access and egress

- Driveways: minimum 12 feet wide clearance, minimum 15 feet vertical clearance for emergency vehicle access

- Create a vegetation buffer on both sides of driveways and access roads

- Ensure home addressing is clearly visible from the road at all times

- If a private gate exists: ensure emergency responders can access without a gate code — install a Knox Box or equivalent

## Maintenance schedule

| Task | Frequency |
| --- | --- |
| Clear Zone 0 of all debris and combustibles | Before and during fire season; after wind events |
| Inspect and clean gutters | 2× per year minimum; after each significant wind event |
| Mow dry grasses in Zones 1–2 | Monthly during dry season |
| Prune dead branches and limbs | Annually; remove dead trees immediately |
| Check irrigation — ensure all Zone 1 plants are well-watered | Weekly during dry season |
| Inspect Zone 2 for dead/dying trees or degraded spacing | Annually |

## Key resources

- CAL FIRE Defensible Space: fire.ca.gov/defensible-space

- CAL FIRE Hazard Severity Zone Maps: osfm.fire.ca.gov/map

- California FireSafe Council: cafiresafecouncil.org

- Sustainable Defensible Space: sdspace.org

- CA Board of Forestry & Fire Protection, Zone 0 rulemaking docket: bof.fire.ca.gov

- IBHS Wildfire Prepared Home: wildfireprepared.org

- CA Department of Insurance, Safer from Wildfires (10 CCR § 2644.9): insurance.ca.gov