# Defensible Space Zone Rules

Source: CAL FIRE — California Department of Forestry and Fire Protection Reference: Public Resource Code 4291 | fire.ca.gov/defensible-space

## Overview

Defensible space is the buffer a homeowner creates between a building and the vegetation, trees, and other plants surrounding it. A well-maintained defensible space gives a home a better chance of surviving a wildfire and gives firefighters a safer environment to defend the structure.

California law requires 100 feet of defensible space for homes located in State Responsibility Areas (SRA). Local jurisdictions may require additional clearance.

## Zone 0 — The Home Ignition Zone (0–5 feet)

Zone 0 addresses the area immediately surrounding the structure. Research consistently shows that most homes ignite not from direct flame contact but from ember accumulation in this zone.

### Goal

Eliminate all combustible material. Create a non-combustible perimeter.

### Surface materials

- Gravel, decomposed granite, or crushed rock (3–4 inch depth)

- Concrete, brick, flagstone, or other masonry

- Pervious concrete pavers

- Sand or bare soil as a minimum baseline

### Vegetation

- No vegetation in Zone 0 is the ideal condition

- If any plants are present, they must be low-growing, well-irrigated, and separated from the structure

- Remove all dead plants, dried leaves, dry grass, and other dead vegetation immediately

### Structures and objects

- Remove all combustible items from under decks and porches — firewood, furniture, doormats, planters

- Clear gutters and roof valleys of all debris and leaf litter

- Enclose open areas beneath elevated decks and porches with non-combustible materials

- Remove wood chip or bark mulch within Zone 0 — mulch can act as an ember bed

### Fencing

- Fencing attached to the home within Zone 0 must be non-combustible: metal, concrete, masonry, fiber cement

- A non-combustible "gate" section of at least 5 feet adjacent to the structure is the minimum if full non-combustible fencing is not possible

### Utilities

- Identify and address gas meter locations relative to the structure

- Clear propane tanks — maintain a 10-foot non-combustible perimeter

## Zone 1 — Lean, Clean, and Green (5–30 feet)

Zone 1 is the area immediately surrounding Zone 0. The goal is to significantly reduce fuel load while maintaining living, well-irrigated plants.

### Goal

Remove dead and dying vegetation. Maintain well-irrigated, fire-resistant plants with appropriate spacing to prevent fire spread from plant to plant and from ground to canopy.

### Vegetation management

- Remove all dead plants, dried grasses, weeds, and leaf litter regularly

- Prune trees to remove dead branches and limbs

- Remove branches that extend over the roof or within 10 feet of the chimney

### Tree spacing

- Maintain a minimum of 10 feet between tree canopies on flat ground

- On slopes, increase spacing — flames travel faster uphill and require greater horizontal separation

- Remove "ladder fuels" — shrubs and smaller trees beneath larger trees that allow fire to climb from ground to canopy

### Shrub spacing

- Space shrubs so that a fire in one shrub cannot easily spread to the next

- On flat ground: space shrubs a minimum of 2 times the height of the shrub apart

- On sloped ground: increase to 3 times the shrub height

### Vertical spacing (limbing up)

- Remove lower branches of trees to a height of at least 6 feet from the ground

- On slopes, increase limb-up height — the steeper the slope, the higher the clearance needed

- Minimum vertical clearance formula: 3× the height of the underlying shrub vegetation

### Ground cover and mulch

- Use composted wood chips (not bark mulch) at a maximum depth of 3 inches

- Keep mulch away from the base of any plant or tree trunk

- Avoid fine, fibrous mulch materials (e.g., gorilla hair) — they ignite rapidly

## Zone 2 — Reduce Fuel (30–100 feet)

Zone 2 addresses the broader landscape beyond the immediate structure zone. The goal is to slow fire spread by reducing overall fuel density.

### Goal

Thin vegetation to slow fire advance and reduce the intensity of a fire approaching the structure. This zone should be low-maintenance relative to Zones 0 and 1.

### Tree spacing — flat sites

- 5–30 ft from structure: minimum 18 feet between tree canopy edges

- 30–60 ft from structure: minimum 12 feet between canopy edges

- 60–100 ft from structure: minimum 6 feet between canopy edges

### Tree spacing — sloped sites

- Slope less than 20%: minimum 20 feet between tree canopies

- Slope 20–40%: minimum 40 feet between tree canopies

- Slope greater than 40%: single trees only — no grouped plantings

### Shrub spacing — sloped sites

- Slope less than 20%: space shrubs 2× their height

- Slope 20–40%: space shrubs 4× their height

- Slope greater than 40%: space shrubs 6× their height, individual plants only

### General Zone 2 maintenance

- Remove all dead and dying trees, shrubs, and vegetation

- Remove branches within 6 feet of the ground on all trees

- Mow or cut annual grasses to a maximum height of 4 inches during dry season

- Address any visible issues — dead or dying trees, collapsed spacing — promptly

## Topography modifiers

Slope, aspect, and local wind exposure significantly increase fire risk. Apply these modifiers on top of standard zone requirements.

| Site condition | Risk increase | Recommended adjustment |
| --- | --- | --- |
| Slope 0–20% | Low | Standard zone distances |
| Slope 20–40% | Moderate | Increase defensible space distances; harden structure |
| Slope 40%+ | High | Maximum hardening; early evacuation planning |
| Canyon or saddle | Very high | Increase Zone 2 distances; limit plantings on uphill side |
| Ridgetop or hilltop | High | Protect all sides equally; increase spacing |
| South/southwest facing | Moderate-high | Higher evapotranspiration — prioritize irrigation |

## Building siting and separation

### Between neighboring structures

- Optimal: 30 feet or more between structures

- If neighbor is also rebuilding: coordinate to place both homes at least 15 feet from shared property line

- If space is constrained: fire-rate the wall(s) closest to the neighboring structure (1-hour minimum)

### Detached structures on property (garages, ADUs)

- Maintain at least 30 feet of separation between structures on the same property

- If less than 30 feet is available, apply additional hardening to the detached structure

### Wildland-Urban Interface (WUI) — open land adjacency

- Maintain at least 50 feet of separation between any structure and open wildland

- If not possible, fire-rate the wall(s) closest to the open land (1-hour minimum)

## Access and egress

- Driveways: minimum 12 feet wide clearance, minimum 15 feet vertical clearance for emergency vehicle access

- Create a vegetation buffer on both sides of driveways and access roads

- Ensure home addressing is clearly visible from the road at all times

- If a private gate exists: ensure emergency responders can access without a gate code — install a Knox Box or equivalent

## Maintenance schedule

| Task | Frequency |
| --- | --- |
| Clear Zone 0 of all debris and combustibles | Before and during fire season; after wind events |
| Inspect and clean gutters | 2× per year minimum; after each significant wind event |
| Mow dry grasses in Zones 1–2 | Monthly during dry season |
| Prune dead branches and limbs | Annually; remove dead trees immediately |
| Check irrigation — ensure all Zone 1 plants are well-watered | Weekly during dry season |
| Inspect Zone 2 for dead/dying trees or degraded spacing | Annually |

## Key resources

- CAL FIRE Defensible Space: fire.ca.gov/defensible-space

- CAL FIRE Hazard Severity Zone Maps: osfm.fire.ca.gov/map

- California FireSafe Council: cafiresafecouncil.org

- Sustainable Defensible Space: sdspace.org