# Zone 0 — Dual Standard: State Mandate vs. Insurance Standard

Proposed update for calfire-zone-rules.md (replaces/expands the existing Zone 0 section) and hazard-flag tagging in assessment-framework.md. Current as of August 2026.

## Why two tracks

Zone 0 (the 0–5 ft area around a structure) is currently governed by two separate systems that do not move on the same timeline and do not require exactly the same things:

|  | Track A — State Mandate | Track B — Insurance Standard |
| --- | --- | --- |
| Authority | CA Board of Forestry & Fire Protection, under PRC § 4291 / AB 3074 | IBHS (Wildfire Prepared Home™), enabled by CA's 10 CCR § 2644.9 |
| Nature | Fire code / legal compliance | Voluntary certification tied to premium discount |
| Status | Draft — not yet enforceable statewide | Active and enforceable today for anyone seeking the designation |
| Stakes if ignored | Citations ($100–$500), lien risk (existing PRC 4291 Zone 1/2 only, via SB 63 inspections) | No discount / no designation — but no penalty |

The tool should report these as two separate outputs, not one merged "Zone 0 status."

## Track A — State Regulatory Mandate

Legal basis: PRC § 4291 has required defensible space (originally 30 ft, expanded to 100 ft in 2005) since the 1970s. AB 3074 (2020) directed the Board of Forestry to write a new 0–5 ft "ember-resistant zone" (Zone 0) standard. Timeline further shaped by SB 504 (2024), AB 1455 (2025), and Executive Order N-18-25, which set a December 31, 2025 adoption deadline.

Current status: That deadline passed without final adoption. The Board's latest draft was released April 17, 2026 (public workshop April 23, 2026). No statewide, enforceable Zone 0 regulation is in effect as of this writing. Existing PRC 4291 Zone 1 (5–30 ft) and Zone 2 (30–100 ft) requirements remain fully in effect and enforceable.

Scope (as drafted): Applies to structures in Very High Fire Hazard Severity Zones (FHSZ), primarily within State Responsibility Areas (SRA); some Local Responsibility Area (LRA) parcels are outside the current draft's scope. Some jurisdictions have moved ahead of the state — notably the City of San Diego, which has an adopted local Zone 0 ordinance already in force, including specific fence/gate exceptions (e.g., existing vinyl fencing allowed under defined conditions).

Draft Zone 0 requirements (subject to change before final adoption):

- Noncombustible ground surface within 5 ft of structure

- No combustible fencing attached to the structure (draft §1298.04(b)(8)–(9))

- Removal of all dead vegetation

- No combustible items attached to or stored against the structure

How the tool should represent this: Label Track A findings as "Anticipated — pending state regulation" rather than "Required," except where a local ordinance (e.g., San Diego) makes it independently enforceable today. Include a jurisdiction check where possible.

## Track B — Insurance Standard

This track has two linked parts: the state rule that forces insurers to offer discounts, and the private standard homeowners get certified against to claim one.

### B1 — 10 CCR § 2644.9, "Safer from Wildfires" (the actual insurance mandate)

Effective October 14, 2022, expanded by AB 1 (Insurance and Wildfire Safety Act), effective January 1, 2026. This regulation requires every admitted California insurer that prices wildfire risk to offer documented discounts across 12 defined mitigation measures — including Zone 0-type actions (noncombustible material within 5 ft, vegetation/debris removal). This is the actual legal lever: once a homeowner documents these actions, insurers are required to reflect it in pricing.

### B2 — IBHS Wildfire Prepared Home™ (the certifiable standard)

Voluntary, insurer-backed designation program. Two tiers:

- Essential (formerly "Base") — ember-focused protection

- Enhanced (formerly "Plus") — adds radiant heat / direct flame protection, builds on Essential

Zone 0 requirement (both tiers, no exceptions):

- Zero combustible material within 5 ft of the structure — no mulch, pine needles, wood chips, vegetation, firewood, propane tanks, trash bins, or combustible furniture

- Acceptable materials: gravel, rock, pavers, concrete, metal furniture

- Essential tier also requires ember-resistant vents (mesh tested to ASTM E2886)

Key rule: designation requires all required actions for the chosen tier — partial mitigation does not qualify.

Financial impact: Documented insurer discounts in the 5–35% range depending on carrier (e.g., USAA 10–20%, Farmers 5–15%, Mercury 10%, Cincinnati up to 20%), plus improved ability to exit the CA FAIR Plan. Designation requires annual maintenance/re-verification.

How the tool should represent this: Label Track B findings as "Actionable today — insurance discount eligible," since this is enforceable and financially live right now, unlike Track A.

## Where the two tracks diverge — flag explicitly

The IBHS standard is stricter than the state's current draft in at least one meaningful way: the state draft allows an exception for well-irrigated, low-growing succulents in Zone 0 (per the existing plant-list logic in socal-plant-list.md); IBHS allows zero vegetation, full stop. A homeowner optimizing for insurance discounts should not rely on the state draft's more lenient language — flag this divergence rather than presenting a single unified rule.

## Recommended change to assessment-framework.md hazard flags

Add a standard field to every Zone 0 flag so recommendations can be filtered/labeled by which track they satisfy:

| Flag | Condition | Standard |
| --- | --- | --- |
| Z0-MULCH | Combustible mulch/bark within 5 ft | BOTH |
| Z0-VEG | Any vegetation within 5 ft | IBHS (state draft allows irrigated succulent exception — flag divergence) |
| Z0-FENCE | Combustible fence attached to structure | BOTH |
| Z0-GUTTER | Debris in gutters/eaves | BOTH |
| Z0-ITEMS | Combustible furniture/items | IBHS (state draft silent on movable items as of current text) |
| Z0-SURFACE | No noncombustible perimeter | BOTH |
| Z0-VENT | Non-ember-resistant vents | IBHS-only (not part of PRC 4291/Zone 0 draft text; state addresses via Ch. 7A building code for new construction only) |

Recommendation output for each flag should then carry two labels instead of one: Regulatory status (Anticipated / Locally enforceable / N/A) and Insurance status (Required for Essential / Required for Enhanced / Not required).

## Sources

- PRC § 4291 (FindLaw); AB 3074 (2020); SB 504 (2024); AB 1455 (2025); Executive Order N-18-25

- CA Board of Forestry & Fire Protection, Zone 0 draft rule, April 17, 2026 (bof.fire.ca.gov)

- IBHS, Wildfire Prepared Home Technical Standard updates, June 2025 and June 2026 (ibhs.org, wildfireprepared.org)

- 10 CCR § 2644.9, "Safer from Wildfires" (insurance.ca.gov)

- AB 1, Insurance and Wildfire Safety Act, effective Jan 1, 2026